| EXTINCTION TYPE | PROPELLENT | EXTINGUISHING | LENG TH JET | TIME OF DISCHARGE | WEIGHT | DIAMETER TANK | HEIGHT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1 C | Nitrogen | Monnex Powder | 12 mt | 37 sec | 143.5 kg | 450mm | 1240mm |

CONSTRUCTIVE CHARACTERISTICS

• Tank: cylindrical steel DD12 UNI EN10111, thickness 30/10, 85 lt

• Type: not pressurized, external bottle 14 lt

• pressare reducer

• Safety valve

• External Treatment: sand-blasting,  epossi-polyester resin paint job

• inner Treatment: phosphating

• Structure: steel

• nitrogen bottle protection Cage, fire hose hook

• distribution  gun

• rubber/twils fire hose, antithrottling spring

• Frontal wheel: ø 150 + brake

• Posterior wheels: ø 400

• Inspection stopper: 2”